|  | Model: 306SA Super Automatic |  |
| --- | --- | --- |

| MANUFACTURED BY: |  | Beaird Poulan Inc. |
| --- | --- | --- |
|  |  | Shreveport Lousiana USA |
| SERIES OR ASSEMBLY NUMBER: |  |  |
| YEAR INTRODUCED: |  | 1970 |
| YEAR DISCONTINUED: |  | 1980 |
| ENGINE DISPLACEMENT: |  | 3.6ci, 59cc |
| NUMBER OF CYLINDERS: |  | 1 |
| CYLINDER BORE: |  | 1.788" |
| PISTON STROKE: |  | 1.437" |
| CYLINDER TYPE: |  | Aluminum alloy with heavy chrome plating |
| INTAKE METHOD: |  | Reed valve |
| MANUFACTURER ADVERTISED H.P.: |  |  |
| WEIGHT : |  | 12 3/4 lbs |
| OPERATOR CONFIGURATION: |  | One man |
| HANDLEBAR SYSTEM: |  | Ridgid |
| CHAIN BRAKE: |  | No |
| CLUTCH: |  | Centrifugal -releases at idle speed |
| DRIVE TYPE: |  | Direct |
| CONSTRUCTION: |  | Die cast magnesium |
| MAGNETO TYPE: |  | Flywheel type magneto |
| CARBURETOR: |  | Tillotson HS-59A |
| MAJOR REPAIR KIT: |  | RK-23HS |
| MINOR REPAIR KIT: |  | DG-5HS |
| AIR FILTER SYSTEM: |  | Flocked wire screen |
| STARTER TYPE: |  | Poulan rewind with oversized pulley |
| OIL PUMP: |  | Auto with manual overide |
| OPERATING RPM: |  | Approx 6500 rpm |
| IGNITION TIMING: |  | Non adjustable |
| BREAKER POINT SETTING: |  | .017" |
| FLYWHEEL/COIL AIR GAP: |  | .012" |
| SPARK PLUG TYPE: |  | Champion CJ-8 |
| SPARK PLUG GAP: |  | .025" |
| CRANKSHAFT MAIN BEARINGS: |  | Roller ball FW side, caged needle on PTO side |
| FUEL TANK CAPACITY: |  | 18oz |
| FUEL OIL RATIO: |  | 16-1 |
| RECOMMENDED FUEL OCTANE: |  | regular |
| MIX OIL SPECIFICATION: |  | Poulan |
| CHAIN PITCH: |  | .325 |
| CHAIN TYPE: |  | Poulan #87 |
| BAR MOUNT PATTERN: |  | Poulan bar and Oregon D176 |
| SHORTEST GUIDE BAR SUPPLIED: |  | 16" |
| LONGEST GUIDE BAR SUPPLIED: |  | 24" |
| COLOUR SCHEME: |  | Poulan lime green with black top covers |
| PAINT CODES: |  |  |
| ILLUSTRATED PARTS LIST: |  |  |
| SERVICE MANUAL: |  |  |

| COMMENTS: |  | Saws below SN541551 were equipped with automatic decompression valves that activated with the throttle lock while saws above SN553210 were equipped with a manual decompression valve. The 306SA's were equipped with a .325 pitch powersharp style chain and a 9 tooth sprocket. The 1978 and latter saws all seem to have been equipped with electronic ign and a quieter tin muffler. |
| --- | --- | --- |