|  | Model: |  |
| --- | --- | --- |

| MANUFACTURED BY: |  | Poulan |
| --- | --- | --- |
| SERIES OR ASSEMBLY NUMBER: |  | 252 |
| YEAR INTRODUCED: |  | 1969 |
| YEAR DISCONTINUED: |  | 1970 |
| ENGINE DISPLACEMENT: |  | 4.5 CI |
| NUMBER OF CYLINDERS: |  | 1 |
| CYLINDER BORE: |  | 2.005 |
| PISTON STROKE: |  | 1.437 |
| CYLINDER TYPE: |  | Aluminum alloy with heavy chrome plating |
| INTAKE METHOD: |  | Reed valve |
| MANUFACTURER ADVERTISED H.P.: |  |  |
| WEIGHT : |  | 17 3/4 LBS |
| OPERATOR CONFIGURATION: |  | One man |
| HANDLEBAR SYSTEM: |  | Ridgid |
| CHAIN BRAKE: |  |  |
| CLUTCH: |  | Automatic Centrifugal (releases at idle speed) |
| DRIVE TYPE: |  | Gear 3.2 : 1 ratio |
| CONSTRUCTION: |  | Die cast Magnesium and sand cast aluminum |
| MAGNETO TYPE: |  | Flywheel type magneto - Mositure and dust proof |
| CARBURETOR: |  | Tillotson all position diaphragm type HS-83A |
| MAJOR REPAIR KIT: |  | RK-23HS |
| MINOR REPAIR KIT: |  | DG-5HS |
| AIR FILTER SYSTEM: |  | Flocked |
| STARTER TYPE: |  | Poulan |
| OIL PUMP: |  | Manual and automatic |
| OPERATING RPM: |  | Approximately 6,500 rpm |
| IGNITION TIMING: |  | Non adjustable |
| BREAKER POINT SETTING: |  | .017 |
| FLYWHEEL/COIL AIR GAP: |  | .012 |
| SPARK PLUG TYPE: |  | Champion CJ-8 |
| SPARK PLUG GAP: |  | .025 |
| CRANKSHAFT MAIN BEARINGS: |  | Flywheel side ball roller, PTO side caged needle bearing |
| FUEL TANK CAPACITY: |  | 27oz |
| FUEL OIL RATIO: |  |  |
| RECOMMENDED FUEL OCTANE: |  |  |
| MIX OIL SPECIFICATION: |  |  |
| CHAIN PITCH: |  | 1/2 |
| CHAIN TYPE: |  | P10 |
| BAR MOUNT PATTERN: |  | Poulan |
| SHORTEST GUIDE BAR SUPPLIED: |  | 15" |
| LONGEST GUIDE BAR SUPPLIED: |  | 31" 13" and 14" bow |
| COLOUR SCHEME: |  |  |
| PAINT CODES: |  |  |
| ILLUSTRATED PARTS LIST: |  |  |
| SERVICE MANUAL: |  |  |

| COMMENTS: |  | Die cast magneisum and sand cast aluminum 74cc gear drive and light weight. Still used the 68-69 200 series ign, top of rear handle kill switch and no throttle lock. |
| --- | --- | --- |